Custom Subnet Masks (also called Variable Length Subnet Masks, or VLSMs)

Step 1: Determine the number of subnets needed

Perhaps 3, for Accounting, R&D, and Legal

Perhaps 4, for Legal, Admin, Marketing, and Customer Service

Perhaps . . . etc.

We will use 6 subnets for this example.

We will assume an assigned Class C network address of 210.199.10.0

Step 2: Determine the number of bits to borrow from the host portion

2n is greater than or equal to the number of subnets required

n = the number of bits to borrow from the host address

In our example, n = 3 will work, since we're trying to obtain 6 subnets. We will 	borrow 3 bits from the host portion of the address. This is binary 111.

Step 3: Determine the subnet mask

From the host portion of the address, n bits from the left become 1's

Step 4: Determine the maximum number of hosts per subnetwork

h = the number of host bits

2h - 2 = maximum number of hosts per network

Step 5: Determine the subnetwork addresses for each subnet

What is the binary value of the lowest bit in the network portion of the subnet?

Example: 255.255.255.224  = 11111111.11111111.11111111.11100000

The underlined bit is the lowest bit in the network portion of the subnet.

It has a binary value of 32.

The subnet addresses will be all multiples of 32.

Remember that subnet address can't be all ones or all zeroes.

So we have:

210.199.10.0			210.199.10.128

210.199.10.32			210.199.10.160

210.199.10.64			210.199.10.192

210.199.10.96			210.199.10.224

Step 6: Determine the address ranges for each subnetwork

Using the second subnet, the first host address is 210.199.10.33

The last host address is 210.199.10.62

Step 7: Determine the broadcast address for each subnetwork

The broadcast address is the last address in the range.

For 210.199.10.32, the broadcast address is 210.199.10.63

# List of networks

### for the 210.199.10.0 network with the subnet mask 255.255.255.224

| Network | Hosts |  | Broadcast Address |
| --- | --- | --- | --- |
|  | from | to |  |
| 210.199.10.0 | 210.199.10.1 | 210.199.10.30 | 210.199.10.31 |
| 210.199.10.32 | 210.199.10.33 | 210.199.10.62 | 210.199.10.63 |
| 210.199.10.64 | 210.199.10.65 | 210.199.10.94 | 210.199.10.95 |
| 210.199.10.96 | 210.199.10.97 | 210.199.10.126 | 210.199.10.127 |
| 210.199.10.128 | 210.199.10.129 | 210.199.10.158 | 210.199.10.159 |
| 210.199.10.160 | 210.199.10.161 | 210.199.10.190 | 210.199.10.191 |
| 210.199.10.192 | 210.199.10.193 | 210.199.10.222 | 210.199.10.223 |
| 210.199.10.224 | 210.199.10.225 | 210.199.10.254 | 210.199.10.255 |